Next Actions and No-Load Lab Readiness Audit

Current Repository Review and Practical Test Plan

| Robot | 6-DOF igus ReBeL using CRI/API-level telemetry |
| --- | --- |
| Repository | github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control |
| Integrated review | 1 October 2026 |

Prepared from the current public repository and the supplied research roadmap, mathematical brief, latency review, and manuscript concept.

# Executive decision

| Verdict | CONDITIONALLY READY. The repository can be taken to the lab now for a short, controlled no-load commissioning/dry test. It is close to ready for the full no-load identification campaign, but three small data-integrity/threading risks should be fixed or explicitly cleared by the dry test before collecting the dataset that will support the paper. |
| --- | --- |

| What changed | The 25 Sep roadmap said the next task was to create an identification-ready logger. The current repository now contains that logger, GUI metadata fields, per-run CSV files, raw q/current capture, monotonic receive timestamps, and a standalone quality checker. This is a real step forward from the roadmap snapshot. |
| --- | --- |

| Meaning of no-load | In this document, no-load means payload_g = 0, contact_label = NORMAL, grasp_context = NONE, with no deliberate contact or collision. This is the model-identification dataset, not the later 45-minute thermal validation experiment. |
| --- | --- |

# 1. Repository evidence reviewed

| Repository element | What it provides | Readiness implication |
| --- | --- | --- |
| IDENTIFICATION_DATA_COLLECTION.md | Operator workflow, metadata definitions, CSV schema, raw-telemetry rules, quality-check command. | Step 1 is documented, not merely planned. |
| core/identification_logger.py | Per-run file creation, no-overwrite naming, row schema, sample logging, quality analysis. | Core logging mechanism exists. |
| gui/main_window.py | Identification metadata panel; Start/Stop controls; CRI status callback writes q/current plus derived qdot/qddot. | Operator can collect runs from the existing GUI. |
| cri_lib/cri_controller.py | Status callback receives a monotonic perf_counter timestamp captured at socket receive. | Measured dt can be reconstructed from real arrival times. |
| cri_lib/cri_protocol_parser.py | POSJOINTCURRENT maps joint position and CURRENTJOINTS maps motor currents. | Required raw signals are available through CRI STATUS. |
| tools/check_identification_csv.py | Reports rows, timestamp monotonicity/duplicates, dt distribution, missing values, metadata, and schema. | A post-run gate exists for data quality. |
| config.yaml / live analytics | FF-RLS lambda=0.98, fixed CUSUM and payload baseline remain separate from the identification logger. | No need to redesign VFF/state logic before collecting normal data. |

# 2. Readiness matrix

| Requirement | Status | Evidence / action |
| --- | --- | --- |
| Dedicated experiment CSV | PASS | IdentificationLogger writes to identification_data/ with one file per run. |
| Run/trajectory metadata | PASS | experiment_id, trajectory_id, run_id, commanded_speed, payload_g, contact_label, grasp_context, thermal_condition are stored. |
| Raw joint positions | PASS | q1..q6 come from CRI POSJOINTCURRENT. |
| Raw motor currents | PASS WITH GUARD | current1..6 come from CRI CURRENTJOINTS, but the current callback silently substitutes zeros if fewer than six values are available. |
| Real timestamps | PASS WITH TEST | A monotonic receive timestamp is supplied; multiple complete messages parsed from one socket recv could theoretically share the same timestamp, so duplicate timestamps must be checked in the dry run. |
| Velocity / acceleration | USABLE AS DERIVED SIGNALS | qdot and qddot are finite differences in the live callback. Do not treat them as independent raw measurements; recompute filtered derivatives offline from q and measured dt. |
| No overwrite / traceability | PASS | Run filenames are timestamped and auto-suffixed if a file already exists. |
| Post-run quality check | PASS | Standalone checker reports dt, duplicates, missing values and signal ranges. |
| Receive-thread workload | RISK TO CLEAR | The status callback writes and flushes the CSV every sample. The CRI controller warns that the callback must remain fast because receive processing is blocked while it runs. |
| Thread-safe metadata access | RISK TO FIX | The receive-thread callback calls _get_identification_metadata(), which reads Qt widgets. Metadata should be frozen at Start and reused from the logger rather than reading GUI widgets from the receive thread. |

# 3. Three pre-campaign fixes / gates

## 3.1 Freeze run metadata at Start - do not read Qt widgets in the receive callback

At Start Identification Run, the GUI already creates the IdentificationLogger with a complete metadata snapshot. The callback can therefore write samples without calling _get_identification_metadata() again. This removes a cross-thread Qt access and also makes metadata immutable within a run, which is scientifically preferable.

Recommended behavior: log_sample(..., metadata=None) # use logger metadata frozen at run start

## 3.2 Never silently turn missing current telemetry into valid-looking zeros

The current callback falls back to six zeros if current_joints has fewer than six elements. For identification, that is dangerous because the CSV remains syntactically valid while the target signal is wrong. Gate recording until CURRENTJOINTS has been observed, or write NaN/drop the sample and count it explicitly.

Before enabling Start Identification Run, confirm the controller has received CURRENTJOINTS and six finite current values.

If a current sample is unavailable during a run, write NaN or skip the sample with a logged counter; do not synthesize 0 mA.

The quality checker should fail the run if current channels are missing, all-zero unexpectedly, or contain non-finite values.

## 3.3 Clear the receive-thread logging performance risk

The CRI receive thread invokes the status callback, and the callback writes a CSV row and flushes the file on every sample. This may be acceptable at the actual STATUS rate, but it should be demonstrated rather than assumed. The immediate dry test should inspect dt percentiles and missed/duplicate timing. If timing becomes irregular, move disk I/O to a queue-backed writer thread or batch flushes outside the receive callback.

| Decision rule | If the dry run produces clean, stable timestamps with no duplicate/invalid dt and no connection/STATUS issues, the current synchronous logger is acceptable for the pilot. If it perturbs timing, change to queued asynchronous logging before collecting the research dataset. |
| --- | --- |

# 4. Lab dry test - do this first

Purpose: validate the logger and signal integrity before spending time on a full no-load campaign. Keep this short and conservative.

Preflight. Use the normal robot risk assessment and workspace safeguards. Start with the robot unloaded, no tool contact, and a known safe pose/trajectory. Do not disable or bypass any certified safety function.

Connect and verify telemetry. Confirm live joint positions and all six joint currents are updating. If any current channel is missing or remains unexpectedly zero, stop and investigate before recording.

Record a stationary sample. Set payload_g=0, contact_label=NORMAL, grasp_context=NONE, thermal_condition=COLD (or the actual condition). Record roughly 30-60 s while stationary.

Record one short low-speed motion. Use one already-safe single-joint sweep or short repeatable trajectory with positive and negative motion. Keep the same metadata except trajectory_id/run_id.

Stop and save each run. Confirm that a separate CSV is created and that the run ID/trajectory ID in the filename and rows are correct.

Run the checker immediately. Use: python tools/check_identification_csv.py identification_data/<file>.csv

Inspect the raw file/plots. Check q and current traces visually; verify currents are plausible and not flat/clipped; confirm q changes only as expected.

# 5. Dry-test acceptance criteria

| Check | Accept for pilot campaign | Stop / fix before campaign |
| --- | --- | --- |
| Timestamp order | Strictly increasing timestamps preferred; no negative dt. | Any negative dt or recurrent duplicate timestamps. |
| dt distribution | Reasonably stable for the observed CRI/PC setup; isolated OS jitter documented. | Large irregular gaps, repeated zero dt, or evidence that logging blocks reception. |
| Required signals | q1..q6 and current1..6 finite for essentially all samples. | Missing/non-finite target signals or silent zero substitution. |
| Current behavior | Each channel shows plausible nonzero baseline and motion-related variation where expected. | All-zero channel, unexplained flat line, or obvious saturation/clipping. |
| Metadata | Exactly one correct experiment/trajectory/run/condition value per file. | Metadata changes within a run or wrong labels. |
| Traceability | One file per run; no overwrite; file path captured in lab notes. | Mixed trajectories in one run without a clear boundary. |
| Derived qdot/qddot | May be stored for reference, but will be recomputed offline. | Using raw finite-difference acceleration as the final Model B signal without filtering/validation. |

# 6. Full no-load identification campaign after the dry test passes

The goal is not maximum data volume; it is identifiability, repeatability, and clean held-out runs. Each trajectory should contain both motion directions where safely possible so friction sign effects are observable.

| Block | Suggested content | Why |
| --- | --- | --- |
| Single-joint excitation | For J1-J6, run safe positive/negative sweeps one joint at a time while other joints remain at defined poses. | Separates per-joint velocity/friction and simple gravity effects. |
| Speed diversity | Repeat at lab-approved low, medium, and higher safe commanded speeds. | Excites viscous/friction terms and tests model generality. |
| Run repeats | At least 2-3 separate run IDs per trajectory/speed during the identification phase. | Allows whole-run holdout and avoids adjacent-sample leakage. |
| Multi-joint trajectories | Add several representative repeatable paths after single-joint data are clean. | Tests coupling/generalization and later supports multi-DOF terms. |
| Thermal labeling | Record COLD/WARM condition honestly, but do not yet use the no-load identification campaign to tune VFF. | Preserves context for later analysis while keeping the scientific sequence clean. |

| Run labels | For the first identification campaign use payload_g=0, contact_label=NORMAL, grasp_context=NONE. Keep experiment_id fixed for the campaign, use trajectory_id for the motion recipe, and increment run_id for true independent repeats. |
| --- | --- |

# 7. What happens immediately after data collection

Build deterministic preprocessing. Retain the raw CSV; create separate modeling columns in rad, rad/s, rad/s^2 and A. Use measured dt. Filter qdot before differentiating.

Fit Model A first. Per joint: [1, qdot, tanh(qdot/v_s), sin(q), cos(q)]. Use whole runs for validation. Report train/held-out RMSE, MAE, residual mean/std, condition number, and residual plots.

Fit Model B second. Add reconstructed qddot using exactly the same train/validation split. Keep acceleration only if held-out performance improves meaningfully without noisy/unstable parameters.

Investigate cross-axis terms only after A/B. Use actual ReBeL geometry/data to justify any gravity/coupling features; do not add arbitrary sine combinations.

Freeze the regressor. Only after this decision should the project move to the thermal burn-in, context-gated VFF-RLS, state machine, threshold calibration, and final experiments.

# 8. Do not do yet

Do not redesign the live FF-RLS into VFF-RLS before the normal-current regressor is frozen.

Do not tune collision, CUSUM, soft-contact, stationary, payload, or VFF thresholds on the no-load identification dataset.

Do not treat the existing live 3-parameter FF-RLS as the final physical model.

Do not use the live qddot column as proof that acceleration belongs in the final regressor.

Do not mix payload/collision trials into the first normal-operation identification campaign.

Do not retune thresholds on the final validation test set later.

# 9. Strongly recommended reproducibility additions (not blockers)

Add code_version or git_commit to the run metadata so each dataset can be traced to the exact software revision.

Store a small experiment manifest (robot/controller version, tool configuration, ambient condition notes, operator notes, trajectory definition) beside the CSVs.

Save the quality-check report for every accepted run rather than only viewing it in the terminal.

Optionally add an automatic post-save quality summary in the GUI after Stop & Save Run.

# 10. Ready-to-paste Copilot task for the final pre-lab patch

Repository: dungnguyen15213/6DOF-Rebel-Cobot-Control

Goal: make the existing identification logger safe for the first no-load data-collection campaign without changing FF-RLS, CUSUM, VFF, payload logic, path planning, or the research model.

Required changes:
1) Freeze identification metadata when Start Identification Run is pressed. Do not call Qt widget getters from _on_robot_status_update; let IdentificationLogger use its metadata snapshot for every row.
2) Do not replace missing CURRENTJOINTS telemetry with six zeros. Before recording, require six finite current values / CURRENTJOINTS availability, or write NaN/drop the sample with an explicit counter and warning.
3) Keep raw q and current immutable. qdot/qddot remain derived candidate fields only.
4) Review the receive-thread write path. If per-sample write+flush can block CRI receive processing, implement a bounded queue + writer thread (or safe batched flush) while preserving timestamp order and clean shutdown.
5) Preserve the existing CSV schema and one-file-per-run behavior unless a new field is added for code_version/git_commit.
6) Add/extend tests for: metadata frozen per run, missing current handling, duplicate filenames, monotonic sample order, clean close/disconnect, and quality-check output.

Do not implement VFF-RLS or new anomaly thresholds in this task.

# 11. Final go/no-go decision

| Stage | Decision |
| --- | --- |
| Take repository to lab for short no-load dry test | GO now, using conservative motion and the existing quality checker. |
| Collect the full identification dataset for the paper | GO after the three pre-campaign risks are fixed or the dry test clearly demonstrates they are benign. |
| Start VFF-RLS/state-machine implementation | NO - wait until Model A/Model B and the final regressor are frozen. |
| Start payload/collision final validation | NO - wait for regressor freeze, thermal evidence, VFF/state implementation, and development calibration. |

# Source basis

This document integrates the following sources. Repository statements were checked against the current public GitHub files available on 1 October 2026; project-stage statements from the PDFs retain their original audit dates.

Public GitHub repository: https://github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control

Identification collection guide: https://github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/blob/main/IDENTIFICATION_DATA_COLLECTION.md

Identification logger: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/core/identification_logger.py

GUI and STATUS callback: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/gui/main_window.py

CRI controller: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/cri_lib/cri_controller.py

CRI parser: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/cri_lib/cri_protocol_parser.py

CSV checker: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/tools/check_identification_csv.py

User-supplied roadmap, math brief, latency review, and Research.docx: