Context-Aware Digital Twin for Polymer-Joint Cobots

Research Summary, Scientific Logic, and Manuscript Layout

| Robot | 6-DOF igus ReBeL using CRI/API-level telemetry |
| --- | --- |
| Repository | github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control |
| Integrated review | 1 October 2026 |

Prepared from the current public repository and the supplied research roadmap, mathematical brief, latency review, and manuscript concept.

# Executive summary

| Research goal | Develop a non-invasive supervisory Digital Twin that uses standard CRI/API telemetry to learn normal joint-current behavior, track slow operating drift, and distinguish transient contact from sustained load changes without adding an external force/torque sensor. |
| --- | --- |

| Current gate | The immediate scientific milestone is still normal-current model identification: collect clean no-payload/no-contact data, fit Model A, compare Model B on identical held-out runs, and freeze the regressor before implementing the final VFF-RLS and context state machine. |
| --- | --- |

| Repository update | The public repository is now ahead of the 25 Sep roadmap audit in one important area: it contains a dedicated identification logger, GUI run metadata controls, per-run CSV output, and a quality-check script. This means Step 1 is substantially implemented in code, although successful robot-side data quality still needs to be demonstrated in the lab. |
| --- | --- |

| Claim boundary | The Digital Twin should be framed as an augmenting supervisory and diagnostic layer. It must not be presented as a replacement for the certified E-stop/safety controller or as equivalent to a calibrated external force/torque sensor. |
| --- | --- |

# 1. Research problem and central question

Low-cost polymer-joint cobots are attractive for lightweight automation, but motor-current signatures are not stationary. Temperature-dependent friction, viscoelastic behavior, direction-dependent friction, payload changes, and external contact can all alter current. A fixed threshold therefore risks confusing normal drift with a real interaction event.

Central research question. Can a context-aware Digital Twin, using only CRI/API-level kinematics and motor-current telemetry, maintain an adaptive baseline of normal operation and separate slow drift, transient collision/contact, and payload/grasp-related changes with experimentally defensible accuracy?

## Working scientific hypotheses

H1 - Normal-current model: A compact physically motivated regressor using joint position, velocity/friction terms, and selected gravity terms can predict normal motor current well enough to create a useful residual.

H2 - Acceleration decision: Adding reconstructed acceleration should be kept only if it improves held-out-run prediction without creating unstable or noise-sensitive parameters.

H3 - Adaptive baseline: A residual-dependent VFF-RLS can track gradual thermal/model drift more effectively than a static or fixed-forgetting baseline, provided abrupt interaction events are prevented from being learned as normal.

H4 - Context disambiguation: Stationary and grasp context can separate payload-related gravity shifts from soft contact or collision events, enabling calibrated sensorless payload inference.

# 2. End-to-end research architecture

| Layer | Function | Current status |
| --- | --- | --- |
| Physical robot | 6-DOF igus ReBeL executes trajectories; CRI exposes STATUS position and motor-current telemetry. | Implemented platform |
| Communication / timing | CRI ACK RTT and telemetry-observed motion response are measured as separate datasets. | Method sufficiently mature to move on; final protocol details remain |
| Identification logger | Per-run CSV with timestamp, q1..q6, qdot1..qdot6, current1..6, candidate qddot1..qddot6, and run metadata. | Implemented in current repo; lab quality validation pending |
| Offline preprocessing | Use measured dt, retain raw columns, convert to radians/A, filter velocity, reconstruct candidate acceleration. | Next after data collection |
| Normal-current model | Fit per-joint static regressor; compare Model A vs Model B on held-out runs. | Immediate scientific gate |
| Adaptive estimator | Context-gated VFF-RLS tracks slow drift while freezing adaptation during interaction candidates. | Concept defined; not yet final in code/calibrated |
| Detection / classification | Residual normalization, CUSUM/persistence, stationary/grasp context, collision/soft-contact/payload states. | Baseline CUSUM exists; final state machine not implemented/calibrated |
| Validation | Thermal, collision, soft-contact, and payload experiments with frozen thresholds and held-out final tests. | Not yet complete |

# 3. Mathematical backbone

The Digital Twin is built around one residual: measured current minus predicted normal current. The research sequence deliberately freezes the static regressor first, then adds adaptation and classification.

I_i(k) = phi_i(k)^T theta_i + e_i(k)

## 3.1 Model A - baseline without acceleration

phi_A = [1, qdot_i, tanh(qdot_i / v_s), sin(q_i), cos(q_i)]

Model A represents offset, viscous friction, smooth Coulomb/Stribeck-like direction dependence, and a simple single-axis gravity component. It is intentionally compact and should be fitted first.

## 3.2 Model B - acceleration hypothesis

phi_B = [1, qddot_i, qdot_i, tanh(qdot_i / v_s), sin(q_i), cos(q_i)]

Acceleration is not automatically accepted. Because numerical differentiation amplifies noise and the telemetry sampling interval is non-uniform, qddot must be reconstructed using measured dt and a documented velocity filter. Model B is retained only if held-out evidence justifies it.

## 3.3 Residual and adaptive baseline

r_k = I_k - I_hat_k

theta_hat_k = theta_hat_(k-1) + a_k K_k e^-_k

The adaptation gate a_k is the key context-aware safeguard: slow drift may be learned, while collision/contact candidates and unresolved payload events freeze parameter updates. The planned VFF law changes lambda according to robust residual-energy evidence rather than using the current fixed lambda = 0.98 baseline forever.

## 3.4 CUSUM and context

S+_k = max(0, S+_(k-1) + z_k - kappa); trigger if S+_k > h (and symmetric negative side)

CUSUM should be described as an evidence accumulator on normalized residuals, not as a guaranteed instantaneous collision detector. Final collision/soft-contact behavior must be established by calibrated thresholds, persistence rules, and experimental validation.

## 3.5 Payload inference

At stationary + valid grasp context: r_bar_i = b_i + s_i m -> m_hat = (r_bar_i - b_i) / s_i

Payload inference is deliberately constrained to approximately stationary conditions after a known grasp event, minimizing velocity and inertial ambiguity. Planned calibration masses are 0, 100, 250, and 500 g.

# 4. Current project status - integrated repo and roadmap view

| Workstream | Evidence now | Interpretation |
| --- | --- | --- |
| CRI / STATUS / GUI | Communication, GUI, path execution, current display, command callbacks and status callbacks are present. | Strong engineering foundation |
| Latency methodology | ACK RTT and telemetry-observed motion response are separated; monotonic timing exists. | Proceed with research; final onset rule/statistics still need freezing |
| Identification-ready logging | Dedicated IdentificationLogger, GUI metadata panel, per-run files, non-overwrite naming, raw q/current fields, candidate derivatives, checker script. | Major improvement since 25 Sep; lab dry-run validation is now the next gate |
| Static current model | Roadmap equations defined; current live FF-RLS remains a 3-parameter baseline. | Offline Model A/B evidence not yet shown |
| VFF-RLS | Concept and equations specified in project documents. | Not yet final/validated in current repo |
| Context state machine | State concept and stationary+grasp rule defined. | Not yet implemented/calibrated |
| Thermal/collision/payload validation | Protocols are planned. | Experimental results still missing |

Important status note. The roadmap's 44% overall completion figure was explicitly a milestone-weighted estimate dated 25 Sep 2026. Because the repository has since added the identification data-collection workflow, that percentage should not be reused as a current score unless the progress matrix is re-scored.

# 5. Recommended Q1/Q2 manuscript structure

The Gemini layout is retained, but the wording is tightened to match the project guardrails and the actual evidence sequence.

## 1. Introduction

Problem: fixed or static current thresholds are brittle when normal current changes with operating condition.

Gap: low-cost polymer-joint cobots need context-aware sensorless diagnostics that do not assume a stationary baseline.

Contribution: an adaptive, context-gated Digital Twin using API-level telemetry for normal-current modeling, anomaly classification, and payload inference.

Safety framing: supervisory augmentation only; no certified safety or force/torque equivalence claim.

## 2. System Architecture and Latency Validation

Hardware/software stack: igus ReBeL, CRI controller, PC-based Python/PyQt Digital Twin.

Method 1: application-layer CRI ACK RTT.

Method 2: GUI/command-to-motion response observed in returned STATUS telemetry.

State clearly that RTT/2 is only an estimated one-way delay under approximate symmetry and TX-to-Motion is not pure actuator latency.

Present latency as sufficiently characterized for supervisory diagnostics; do not present final latency performance until the final protocol is frozen and repeated.

## 3. Dynamic Kinematic Modeling and Adaptive Baselines

Data and preprocessing: measured dt, units, filtering, run-based train/validation split.

Model A and Model B comparison, followed by physically justified multi-DOF/cross-axis terms only if supported by geometry/data.

Static least-squares baseline, fixed-FF RLS baseline, and final context-gated VFF-RLS.

Define the a-priori residual and robust residual-energy statistic used to control lambda.

## 4. Context-Aware Anomaly Disambiguation

NORMAL / ADAPTIVE DRIFT: estimator may adapt.

TRANSIENT COLLISION: residual/CUSUM evidence triggers event classification and freezes adaptation.

SOFT CONTACT: sustained in-motion departure without valid payload context.

PAYLOAD / GRASP EVENT: classification only with stationary + valid grasp context; convert calibrated residual shift to mass.

Specify hysteresis, persistence, release conditions, and event priority after calibration.

## 5. Experimental Validation and Results

No-load normal-operation identification dataset and held-out Model A/B decision.

Thermal tracking: controlled cold-to-warm repeated trajectory, approximately 45 min after regressor freeze.

Collision/contact: cold and warm tests; report ROC/precision/recall/F1/confusion matrices as appropriate.

Payload: 0/100/250/500 g calibration and held-out evaluation; report RMSE, MAE, bias, standard deviation/uncertainty.

Compare static baseline vs fixed-FF RLS vs VFF-RLS on the same replay/test data.

## 6. Discussion and Limitations

What can and cannot be inferred from motor current alone.

Effect of non-uniform sampling and reconstructed derivatives.

Dependence on robot configuration, pose, trajectory, temperature proxy, and payload geometry.

Why the system augments, rather than replaces, certified safety functions and external force/torque sensing when those are required.

## 7. Conclusion

Summarize the software-only supervisory contribution and its experimentally demonstrated scope.

Use the phrase: sensorless contact and payload inference without an external force/torque sensor.

Avoid broader capability claims than the validated test matrix supports.

# 6. Claim-to-evidence map

| Paper claim | Required evidence | Do not claim before |
| --- | --- | --- |
| Normal-current model is predictive | Per-joint held-out RMSE/MAE, residual mean/std, condition number, residual plots on whole held-out runs. | Model A/B comparison is complete |
| Acceleration improves the model | Same split, same runs, stable parameters, meaningful held-out improvement. | Model B passes decision gate |
| Thermal drift motivates adaptation | Cold-to-warm repeated trajectory showing systematic residual/baseline drift. | Thermal experiment is complete |
| VFF-RLS outperforms static/fixed-FF baseline | Replay or test-set comparison on identical datasets with frozen design. | VFF implementation and development calibration are frozen |
| Collision/contact classification works | Cold/warm event trials and classification metrics on held-out test data. | Thresholds are frozen and final validation is complete |
| Payload can be inferred sensorlessly | Calibration at known masses plus held-out mass errors and uncertainty. | Stationary+grasp calibration and test are complete |

# 7. Research sequence from today to paper

Validate the identification logger in the lab. Run a short no-load dry test and confirm timestamps, currents, metadata, and file integrity.

Collect no-load normal-operation runs. Use repeatable single-joint and multi-joint trajectories at multiple safe speeds; hold out entire runs.

Preprocess offline. Retain raw values, use measured dt, filter velocity, reconstruct candidate acceleration, inspect signal quality.

Fit Model A, then Model B. Make an evidence-based acceleration decision and freeze the simple baseline regressor.

Evaluate cross-axis gravity/coupling only if justified. Freeze the final regressor before adaptive-estimator work.

Run thermal baseline experiment. Quantify drift that motivates adaptation.

Implement and replay-test context-gated VFF-RLS. Keep static and fixed-FF baselines for direct comparison.

Implement and calibrate context state logic. Freeze thresholds using development data only.

Run final validation. Cold/warm collision, soft-contact, payload trials; no retuning on final test data.

Integrate results into the manuscript. Every technical claim must map to an implemented equation and a measured experiment.

# 8. Manuscript-safe wording guide

| Prefer | Avoid |
| --- | --- |
| "supervisory Digital Twin that augments existing safety functions" | "replaces the E-stop / certified safety controller" |
| "sensorless contact and payload inference without an external force/torque sensor" | "equivalent to a force/torque sensor" |
| "CRI ACK RTT" and "estimated one-way delay = RTT/2 under approximate symmetry" | "measured one-way latency = RTT/2" |
| "telemetry-observed TX-to-Motion response" | "pure actuator latency" |
| "CUSUM accumulates evidence of residual departure" | "CUSUM instantly detects every collision" |

# Source basis

This document integrates the following sources. Repository statements were checked against the current public GitHub files available on 1 October 2026; project-stage statements from the PDFs retain their original audit dates.

Public GitHub repository: https://github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control

Current identification collection guide: https://github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/blob/main/IDENTIFICATION_DATA_COLLECTION.md

Current identification logger implementation: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/core/identification_logger.py

Current GUI/status callback: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/gui/main_window.py

User-supplied roadmap:

User-supplied next-steps/math brief:

User-supplied latency methodology review:

User-supplied Research.docx / Gemini manuscript concept: