Summaries about the Cobot AI Algorithm Controlling Project:

At first, I decide to just install the indicator on the 6DOF Cobot to test on a reference point or line to measure the precision. The AI agent (Gemini Pro) suggest to use the laser to point a laser point on a reference paper install nearby. With this option, we require a camera can detect the point on the paper. This can be a good approach to testing a precision of the AI algorithm controlling. So, in this robot controlling algorithm project, we can do both.

Hardware requirements:

| Hardware | Status | Picture | Note |
| --- | --- | --- | --- |
| The Cobot – IGUS Rebel 6-DOF | Available in Innovation Hub and Maker Space |  |  |
| Dial Indicator | Available in Innovation Hub and Maker Space |  |  |
| Indicator Setup | Required to design and do 3d printing. |  | PETG filament required. Estimated: ~1 kg/ filament (don’t have any stock yet) |
| Laser Pointer | Required to buy a new one |  | Cheap rice is enough |
| Laser Pointer Setting: + Camera to detect the point on the paper + A ruler grid paper as a target for Laser point (Laser Dot) | Required to buy a new camera Required to design and printing the reference paper |  | Camera price and type need to be researched carefully. |

Task to do:

- Build a software to simulation and control the Cobot normally. It will be used to be the simulation environment and working at a Digital Twin for future controlling.

- Define the AI – based controlling definition that we want to do with our project. For this definition, I have put on the AI and get the response like this [2].

My final words:

- In this case, I need to discuss with you because I don’t know which idea that we should choose to control the robot for now. Next week, I think we should meet one more time in next week to get the final idea for this research after weekends. Of course, I will do properly research in this weekend to finalize the idea into a professional research. This AI suggestion below can be the reference but I don’t think it’s good to follow. Although, these parameter could be considered to choose at our final analysis parameters.

Analysis Parameter for comparision

| Parameter | What it tells you |
| --- | --- |
| Pose Repeatability | Does the algorithm output the exact same joint angles every time for the same target? |
| Absolute Accuracy | Can the algorithm hit a brand new coordinate it has never seen before? |
| Inference Latency | Is the AI faster at calculating complex poses than traditional calculus? |
| Joint Smoothness | Is the AI causing micro-stutters that could damage the ReBeL's polymer gears? |
| Failure Rate | How safe and reliable is the AI for industrial use? |

[1] Response of AI for planning a project use Indicator and Laser point.

## 1. Hardware & Software Requirements

### Hardware

- The Robot: igus ReBeL 6-DOF cobot with power supply and Ethernet cable.

- Safety: Physical Emergency Stop (E-Stop) button connected and within reach.

- Compute: A PC running Ubuntu 22.04 or 24.04 with a dedicated GPU (e.g., NVIDIA RTX series) for AI training and fast inference.

- Indicator Setup: Dial indicator (0.01mm precision), 3D-printed breakaway tool-flange mount, and a heavy, flat metal calibration block.

- Laser Setup: Standard laser pointer (pen style or diode module), 3D-printed mount, and A3/A4 millimeter graph paper taped to a rigid, flat board.

### Software

- Core: ROS 2 (Humble or Jazzy) and MoveIt 2.

- Simulation: Gazebo Harmonic and RViz2.

- AI Stack: Python 3, PyTorch (or TensorFlow), and NumPy.

## 2. The 2-Month Master Plan (8 Weeks)

### Phase 1: Baseline Setup & Traditional Testing (Weeks 1 - 2)

- Week 1: Digital Twin & Hardware Prep

  - 3D print both the indicator breakaway mount and the laser pointer mount.

  - Connect the ReBeL to the PC via the ros2-igus-rebel driver.

  - Verify the robot matches its digital twin in RViz2.

- Week 2: The Baseline Math Experiments

  - Run the Dial Indicator Test (Repeatability) using traditional Inverse Kinematics (MoveIt 2 TRAC-IK). Loop the robot to the metal block 50 times and record the dial readings.

  - Setup the graph paper and run the Laser Pointer Test (Accuracy). Feed 20 random coordinates to MoveIt 2, mark where the laser lands, and log the data.

### Phase 2: AI Development & Training (Weeks 3 - 4)

- Week 3: Data Generation

  - Write a script in ROS 2 simulation to generate 100,000 valid pairs of Cartesian coordinates $(x, y, z)$ and their corresponding 6 joint angles.

  - Design your Neural Network (Neural IK) in PyTorch.

- Week 4: AI Training

  - Train the Neural Network using the generated dataset.

  - Validate the model offline (feed it coordinates on your PC and check if the predicted joint angles make mathematical sense before connecting the physical robot).

### Phase 3: AI Integration & Testing (Weeks 5 - 6)

- Week 5: The Software Bridge

  - Write a Python node in ROS 2 that takes the output of your AI model (the predicted joint angles) and sends them to the ReBeL's joint trajectory controller.

- Week 6: The AI Dial Indicator Test

  - Mount the dial indicator. Keep speed at 10% and hand on the E-Stop.

  - Ask the AI to calculate the joint angles for the measurement point 50 times.

  - Record the indicator readings to see if the AI's predictions fluctuate (jitter) compared to the math baseline.

### Phase 4: Final Testing & Analysis (Weeks 7 - 8)

- Week 7: The AI Laser Pointer Test

  - Swap to the laser pointer mount.

  - Feed the AI the same 20 random coordinates used in Week 2.

  - Execute the movements and measure the distance between the target and the actual laser dot.

- Week 8: Data Crunching & Reporting

  - Compile all data into tables and graphs.

  - Analyze where the AI succeeded and where traditional math performed better.

## 3. Parameters to Collect & Analyze

To write a strong technical report or paper at the end of this project, you need to track these specific parameters during your testing:

| Parameter | How to calculate it | What it tells you |
| --- | --- | --- |
| Pose Repeatability | Standard deviation of the dial indicator readings across 50 attempts. | Does the algorithm output the exact same joint angles every time for the same target? |
| Absolute Accuracy | $d = \sqrt{(y_t-y_a)^2 + (z_t-z_a)^2}$ (Distance between target and actual laser dot). | Can the algorithm hit a brand new coordinate it has never seen before? |
| Inference Latency | Time in ms from inputting the coordinate to outputting the joint commands. | Is the AI faster at calculating complex poses than traditional calculus? |
| Joint Smoothness | Derivative of joint acceleration recorded via ROS 2 topics (rad/s^3). | Is the AI causing micro-stutters that could damage the ReBeL's polymer gears? |
| Failure Rate | Percentage of times the algorithm outputs an impossible or self-colliding pose. | How safe and reliable is the AI for industrial use? |

ISO 9409-1 pattern**

[2] Definition of AI Based 6dof Cobot Controlling?

When we talk about "AI-based 6-DOF cobot controlling," we are talking about replacing or assisting traditional mathematical controllers (like PID controllers, calculus, and geometry) with Machine Learning models.

In the robotics industry right now, AI-based control is divided into four main categories. If you are doing a project to compare algorithms, you will likely choose one from Category 1 or Category 2.

Here are the specific AI algorithms used to control a 6-DOF cobot:

### 1. Reinforcement Learning (RL) Control

The "Trial and Error" Approach

This is the most popular AI method for direct, low-level robot control. You put the digital twin of your ReBeL cobot in a simulation (like Gazebo or Isaac Sim) and let it try to reach a target point millions of times. It gets a "reward" when it gets close and a "penalty" if it crashes.

- The Algorithms:

  - PPO (Proximal Policy Optimization): The absolute industry standard right now. It is highly stable and great for continuous robot movements.

  - SAC (Soft Actor-Critic): Very good at exploring its environment and learning smooth trajectories.

  - DDPG (Deep Deterministic Policy Gradient): An older but very effective algorithm specifically designed for continuous joint control.

- How it works for your project: You feed the AI the target $(x, y, z)$ coordinate and the current joint angles. The AI directly outputs the joint velocities to move the arm to the target.

### 2. Neural Inverse Kinematics (Neural IK)

The "Math Approximation" Approach

Traditional Inverse Kinematics (IK) uses complex matrices and trigonometry to figure out how to bend the 6 joints to reach a coordinate. Sometimes, this math is slow or gets stuck in a "singularity."

- The Algorithms:

  - Deep Neural Networks (MLP - Multi-Layer Perceptrons): You train a standard, deep neural network on hundreds of thousands of successful movements.

- How it works for your project: Instead of doing math, the AI just memorizes the relationships. You input $(x,y,z)$, and the AI instantly predicts the 6 joint angles ($\theta_1, \theta_2, \theta_3, \theta_4, \theta_5, \theta_6$) in milliseconds.

### 3. Imitation Learning / Behavioral Cloning

The "Monkey See, Monkey Do" Approach

Instead of letting the robot learn by trial and error, a human uses a joystick or VR controller to move the physical robot to do a task (like picking up a block) 50 times. The AI watches the cameras and the joint data and learns to copy the human.

- The Algorithms:

  - Diffusion Policies: The hottest algorithm in robotics right now (used by Toyota and MIT). It creates incredibly smooth, human-like, continuous movements.

  - ACT (Action Chunking with Transformers): Uses the same technology behind ChatGPT to predict the next physical movement of the robot.

### 4. Vision-Language-Action (VLA) & Code-as-Policy

The "High-Level Brain" Approach

This doesn't control the individual motors directly. Instead, it acts as the "manager."

- The Algorithms:

  - LLMs (Large Language Models): Models like Gemini 1.5 Pro or GPT-4o.

  - VLA Models: RT-2 (Robotics Transformer 2) or OpenVLA.

- How it works: You type, "Move the laser pointer to the top right corner of the paper." The AI understands the text, looks at a camera feed, and writes a Python script that commands ROS 2 to move the robot.

### Which one should you use for your project?

Since your project is about testing precision (Laser Pointer) and repeatability (Dial Indicator) against traditional baseline math, you should choose:

Reinforcement Learning (PPO algorithm) or Neural IK (MLP algorithm).

These are the best for precision point-to-point targeting. Imitation Learning is better for messy tasks (like folding laundry), and LLMs are better for high-level reasoning, not sub-millimeter precision.

If you want to start with the easiest one to program, Neural IK using PyTorch is a fantastic and highly scientific starting point. Are you familiar with PyTorch, or would you like to see how to structure the input/output layers for a 6-DOF robot?