Context-Aware Digital Twin for Polymer-Joint Cobots

Step-by-Step Research & Implementation Roadmap

Repository: dungnguyen15213/6DOF-Rebel-Cobot-Control
Robot: 6-DOF igus ReBeL using CRI/API-level telemetry
Roadmap audit date: 25 September 2026

| 44% Overall project-to-paper completion | ~70% Engineering/platform foundation | ~25% Core research novelty + validation |
| --- | --- | --- |

Interpretation: The robot communication, GUI, logging, baseline FF-RLS, CUSUM, payload prototype, and latency infrastructure provide a strong platform. The main unfinished work is the scientific contribution: final regressor identification, true VFF-RLS, context-state logic, calibration, and experimental validation. Percentages are milestone-weighted planning estimates, not measured labor hours.

# 1. Executive status

Current decision: Do not modify the live Digital Twin RLS/state machine yet. First make the repository identification-ready, collect a clean normal-operation dataset, and compare candidate current regressors offline. Only after the regressor is selected should VFF-RLS and the context-aware state machine be implemented.

Immediate next milestone: Identification-ready dataset and offline Model A / Model B comparison.

# 2. What has already been done

CRI communication and STATUS telemetry pipeline are established.

PyQt-based Digital Twin / GUI foundation exists and launches from main.py.

Path planning/export/execution infrastructure exists; the controller owns timing, acceleration, and velocity shaping.

Diagnostic logging exists and already records time, 6 joint angles, 6 motor currents, 6 velocities, and 6 accelerations.

Fixed-forgetting-factor RLS exists with lambda = 0.98 and a 3-parameter configuration.

Two-sided CUSUM baseline exists with configurable k and h.

A basic payload estimator exists with filter, sensitivity gain, and attach threshold configuration.

Latency methodology has been separated conceptually into CRI ACK RTT and telemetry-observed motion response.

Research safety framing is established: supervisory Digital Twin only; no equivalence claim to an external force/torque sensor.

VFF-RLS mathematics V1 and context-state logic V1 have now been defined conceptually, but are not yet implemented or calibrated.

# 3. What is not yet complete

Final physically defensible current regressor phi(q, q_dot, q_ddot).

Experimental decision on whether reconstructed acceleration improves held-out current prediction.

Cross-axis / multi-DOF gravity coupling model justified from robot geometry or data.

True residual-dependent VFF-RLS in code.

Robust residual normalization and slow residual-energy statistic for VFF control.

Adaptation freeze/gating during collision, soft-contact, or pending payload decisions.

Context-aware state machine implementation and hysteresis / timing rules.

Stationary threshold, grasp context, soft-contact threshold, collision threshold, and CUSUM threshold calibration.

Payload sensitivity and empty-state calibration at planned masses 0 / 100 / 250 / 500 g.

Cold-vs-warm collision/contact experiments and final confusion matrices / ROC / precision / recall / F1.

Final payload RMSE / MAE / bias / standard deviation.

Final paper results and conclusions.

# 4. Repository audit for the next step

Verdict: YES - the public repository is suitable as a starting point for the next model-identification step, with a few preparation checks. There is no need to rebuild the GUI, CRI layer, or basic logger before beginning offline identification.

| Repo evidence | What it shows | Readiness | Action |
| --- | --- | --- | --- |
| config.yaml | FF-RLS lambda=0.98; n_params=3; fixed CUSUM k/h; simple payload gain/threshold | Partial | Keep as baseline; do not call it VFF. |
| settings.py | Typed settings support only current FF-RLS, CUSUM, payload, logging parameters | Partial | Later add VFF/state-machine settings after equations/calibration are frozen. |
| robot_diagnostics.csv | Time, q1..q6, current1..6, velocity1..6, acceleration1..6 are present | Good | Use this schema as the base identification dataset. |
| robot_diagnostics.csv timestamps | Sampling intervals are non-uniform in the checked log | Important | Always use measured dt; never assume fixed sampling for differentiation/filtering. |
| QUICK_START / PATH_EXPORT_GUIDE | Path execution exists and controller performs motion timing/profile shaping | Good | Use repeatable commanded trajectories for identification; log actual STATUS telemetry. |
| main.py | Application entry point and GUI wiring are straightforward | Good | No GUI work needed for the immediate model-identification milestone. |

Repository limitation of this audit: The public GitHub root and raw top-level files were accessible, but GitHub directory-tree crawling was restricted in this session. Therefore this audit verifies the top-level configuration, startup, path-planning documentation, and checked diagnostic output, but it does not claim a line-by-line review of every file under core/, gui/, hardware/, and tests/.

# 5. Small repository preparation before data collection

5.1 Preserve raw telemetry - Confirm the experimental CSV contains the raw CRI STATUS joint position/current fields before any RLS or anomaly-processing transformation. Keep processed signals in additional columns rather than overwriting raw values.

5.2 Use real timestamps - Use monotonic/sample timestamps and compute dt[k] = t[k] - t[k-1]. The checked CSV is not uniformly sampled.

5.3 Add run metadata - At minimum record experiment_id, trajectory_id, run_id, commanded speed, payload_g, contact_label, grasp_context, and cold/warm condition. For the first identification dataset set payload_g=0 and contact_label=NORMAL.

5.4 Freeze live analytics during identification - Do not use adaptive RLS output as the target. Fit the offline model against measured motor current. Existing FF-RLS/CUSUM can continue running for display, but they must not alter the logged raw telemetry.

5.5 Keep units explicit - Current log uses degrees, deg/s, deg/s^2, and mA. Convert to a consistent modeling convention before fitting, preferably rad, rad/s, rad/s^2, and A, while retaining original columns.

5.6 Treat existing acceleration as unverified - The CSV contains acceleration, but before using it in Model B, compare it against a filtered derivative of velocity computed with actual dt. Do not assume the current acceleration column is sufficiently low-noise for identification.

# 6. Step-by-step TODO list

STEP 1 - Make an identification-ready logger/dataset

Status: NEXT    |    Timing: 1-2 lab sessions

Create a dedicated experiment output file instead of reusing an arbitrary diagnostics file.

Required columns: timestamp, q1..q6, qdot1..qdot6, current1..current6. Keep qddot1..qddot6 as optional/candidate columns.

Add run_id / trajectory_id / condition metadata.

Verify no missing values, duplicated timestamps, unit ambiguity, or current clipping.

Confirm actual dt distribution from timestamps.

Definition of done: one CSV can be loaded offline and each sample can be assigned to a specific trajectory/run.

STEP 2 - Collect clean normal-operation identification data

Status: NEXT    |    Timing: 1-2 lab sessions

No payload, no intentional contact, no collision.

Record positive and negative motion for all joints.

Use low, medium, and higher safe commanded speeds.

Include single-joint excitation and representative multi-joint trajectories.

Repeat trajectories as separate runs so entire runs can be held out for validation.

Definition of done: at least several independent runs with sufficient motion diversity and clean labels.

STEP 3 - Build offline preprocessing

Status: AFTER STEP 2    |    Timing: Notebook/script

Convert units to A and radians-based kinematics.

Use measured dt.

Filter qdot before numerical differentiation.

Generate a candidate qddot signal with a documented filter/differentiator.

Plot q, qdot, qddot, current, and dt for quality control.

Definition of done: preprocessing is deterministic and reproducible from raw CSV.

STEP 4 - Fit Model A

Status: AFTER STEP 3    |    Timing: Offline

Use phi_A = [1, qdot_i, tanh(qdot_i/v_s), sin(q_i), cos(q_i)].

Fit parameters per joint with numerically stable least squares (not explicit matrix inversion).

Compute train and held-out RMSE, MAE, residual mean/std, and regression condition number.

Definition of done: Model A has per-joint validation metrics and residual plots.

STEP 5 - Fit Model B and decide whether acceleration stays

Status: AFTER STEP 4    |    Timing: Offline

Use phi_B = [1, qddot_i, qdot_i, tanh(qdot_i/v_s), sin(q_i), cos(q_i)].

Use the same train/validation split as Model A.

Keep acceleration only if it produces meaningful held-out improvement without unstable/noisy parameters.

Definition of done: evidence-based A-vs-B decision for each joint and one chosen baseline regressor.

STEP 6 - Investigate multi-DOF gravity/coupling terms

Status: AFTER STEP 5    |    Timing: Model design

Use actual ReBeL kinematics/geometry to propose cross-axis gravity terms; do not add arbitrary sine combinations.

Add only terms that are physically interpretable and identifiable from the data.

Repeat held-out comparison against the selected simpler model.

Definition of done: final regressor phi is frozen for all later estimator comparisons.

STEP 7 - Thermal/warm-up baseline experiment

Status: AFTER REGRESSOR FREEZE    |    Timing: ~45 min experiment

Repeat a controlled trajectory from cold start through warm operation.

Keep the selected static model fixed first.

Measure residual drift versus elapsed time/temperature proxy.

Definition of done: quantitative evidence that the baseline changes enough to motivate adaptation.

STEP 8 - Implement context-gated VFF-RLS

Status: AFTER STEP 7    |    Timing: Code + replay test

Implement normalized a-priori residual, robust slow residual-energy statistic, lambda_min/lambda_max law, and lambda smoothing.

Implement adaptation gate so collision/contact candidates freeze parameter updates.

Retain fixed-FF RLS as a baseline selectable by config.

Definition of done: static, fixed-FF, and VFF estimators can replay the same dataset and produce comparable outputs.

STEP 9 - Implement the context-aware state machine

Status: AFTER VFF    |    Timing: Code + replay test

States: NORMAL/ADAPTIVE DRIFT, TRANSIENT COLLISION, SOFT CONTACT, PAYLOAD/GRASP EVENT.

Payload classification permitted only under stationary + grasp context.

Add persistence timers, release thresholds, hysteresis, and event priorities.

Definition of done: deterministic state transitions can be tested from prerecorded logs.

STEP 10 - Calibration experiments

Status: AFTER STATE MACHINE    |    Timing: Lab

Estimate residual scale sigma_e per joint from normal data.

Calibrate stationary velocity thresholds, CUSUM parameters, collision/soft-contact thresholds, timing windows, and VFF bounds/time constants.

Calibrate payload sensitivity using 0/100/250/500 g and empty-state references.

Freeze thresholds using development data only.

Definition of done: all thresholds in config are experimentally justified and frozen before final test.

STEP 11 - Final validation experiments

Status: AFTER CALIBRATION    |    Timing: Lab

Static baseline vs fixed-FF RLS vs VFF-RLS.

Cold and warm collision trials.

Sustained soft-contact trials.

Payload/grasp trials with planned masses.

Compute ROC/precision/recall/F1/confusion matrices and payload RMSE/MAE/bias/std.

Definition of done: test-set results are complete and no threshold is retuned on final test data.

STEP 12 - Manuscript integration

Status: FINAL    |    Timing: Writing

Update Section 3 with final regressor + VFF equations.

Update Section 4 with exact state-machine guards/transitions.

Report latency datasets separately unless explicitly correlated.

Use the phrase sensorless contact and payload inference without an external force/torque sensor.

State clearly that the Digital Twin augments, not replaces, certified safety functions.

Definition of done: every technical claim maps to implemented equations and an experiment/result.

# 7. Progress matrix

| Workstream | Weight | Done credit | Workstream completion | Status |
| --- | --- | --- | --- | --- |
| Research scope & safety framing | 5% | 5% | 100% | Complete/strong |
| CRI / STATUS / GUI foundation | 15% | 14% | 93% | Complete/strong |
| Logging & latency infrastructure | 10% | 8% | 80% | Complete/strong |
| Baseline FF-RLS / CUSUM / payload | 10% | 7% | 70% | In progress |
| Final regressor / model identification | 15% | 3% | 20% | In progress |
| Context-gated VFF-RLS | 12% | 3% | 25% | In progress |
| Context state machine | 12% | 3% | 25% | In progress |
| Calibration experiments | 8% | 0% | 0% | Not started |
| Validation experiments | 8% | 0% | 0% | Not started |
| Results & manuscript finalization | 5% | 1% | 20% | In progress |

Overall milestone-weighted completion = 44%. This number intentionally gives substantial weight to the still-unfinished scientific novelty and validation. It should be updated after each milestone rather than treated as a precise productivity metric.

# 8. Exact next actions - do these now

Do not change the live RLS formula yet.

Create/confirm a dedicated identification CSV logger with raw time, q, qdot, current and run metadata.

Run one short dry test and inspect timestamp spacing, units, missing data, and current saturation.

Collect the first no-payload/no-contact trajectory set with separate run IDs.

Create an offline notebook/script that loads those runs and constructs Model A.

Add filtered acceleration and Model B only after Model A works.

Return with the dataset/notebook outputs; then freeze the regressor before VFF implementation.

# 9. Planned repository changes after the regressor is frozen

These are intentionally deferred so Copilot/code changes do not drive the research design:

Extend config.yaml/settings.py with VFF bounds/time constants, residual normalization settings, gating thresholds, and state-machine timers.

Add a model/regressor module with a stable interface for static, fixed-FF, and VFF estimators.

Add context/state-machine module independent from the GUI.

Add replay-based unit/integration tests using recorded telemetry.

Add experiment metadata and versioned calibration files.

Keep raw telemetry logs immutable; write derived analytics to separate outputs.

# 10. Repository evidence checked

Repository root: https://github.com/dungnguyen15213/6DOF-Rebel-Cobot-Control

config.yaml: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/config.yaml

settings.py: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/settings.py

main.py: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/main.py

QUICK_START.md: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/QUICK_START.md

PATH_EXPORT_GUIDE.md: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/PATH_EXPORT_GUIDE.md

PATH_EXPORT_REFERENCE.md: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/PATH_EXPORT_REFERENCE.md

robot_diagnostics.csv: https://raw.githubusercontent.com/dungnguyen15213/6DOF-Rebel-Cobot-Control/main/robot_diagnostics.csv

# 11. Research guardrails to keep throughout the project

Do not claim that the Digital Twin replaces the E-stop or certified safety controller.

Do not claim equivalence to a force/torque sensor.

Use: sensorless contact and payload inference without an external force/torque sensor.

Do not call RTT/2 measured one-way latency; call it estimated one-way communication delay under an approximate symmetry assumption.

Do not call telemetry-observed motion response pure actuator latency.

Do not let Copilot invent the VFF law, regressor physics, state-machine guards, or calibration thresholds.

Do not tune final thresholds on the test set.