PROJECT: Context-Aware Digital Twin for Polymer-Joint Cobots

REPOSITORY:
dungnguyen15213/6DOF-Rebel-Cobot-Control

ROBOT:
6-DOF igus ReBeL cobot using CRI/API-level telemetry.

TARGET PAPER:
“An Adaptive Digital Twin Framework for Context-Aware Anomaly Classification and Sensorless Payload Identification in Polymer-Joint Cobots.”

CORE RESEARCH OBJECTIVE:
Develop a non-invasive PC-based Digital Twin supervisory layer capable of distinguishing normal thermal/friction drift, transient collisions, sustained soft contact, and payload/grasp-induced load changes using only normal robot API telemetry.

IMPORTANT SAFETY/CLAIM FRAMING:
The Digital Twin augments the cobot's existing certified safety system. It does NOT replace the robot's E-stop or certified safety controller.

Do not claim equivalence to a force/torque sensor. Use:
“sensorless contact and payload inference without an external force/torque sensor.”

CURRENT REPO CAPABILITIES:

- CRI communication

- robot STATUS telemetry

- GUI Digital Twin

- current monitoring

- fixed-forgetting-factor RLS

- two-sided CUSUM

- basic payload estimator

- analytics/logging

- latency methodology and supporting infrastructure

IMPORTANT CURRENT MODEL:
The current RLS is NOT true VFF-RLS.

Current forgetting factor is effectively constant, e.g.:

lambda = 0.98

Current approximate live per-joint regressor:

phi_i = [
q_dot_i,
sign(q_dot_i),
sin(q_i)
]

Therefore the current repository does NOT yet support manuscript claims of:

- residual-dependent lambda(t)

- true VFF-RLS

- live acceleration term in RLS

- cross-axis gravity coupling

- complete multi-DOF dynamic model

CURRENT CUSUM:
Two-sided CUSUM exists and operates on RLS residual:

e(t) = I_measured - I_predicted

CUSUM is considered a valid starting point for transient/change detection.

CURRENT PAYLOAD ESTIMATOR:
A simple payload estimator exists based on parameter/gravity-coefficient changes.

However:

- empty-state parameters require experimental calibration,

- sensitivity must be experimentally calibrated,

- payload inference is NOT currently fully gated by stationary/grasp context.

MAJOR RESEARCH FEATURE STILL TO IMPLEMENT:
Context-aware state machine.

Target classes/states:

NORMAL / THERMAL DRIFT
TRANSIENT COLLISION
SOFT CONTACT
PAYLOAD / GRASP EVENT

Core disambiguation principle:

Payload inference should only be allowed under valid kinematic/task context, especially approximately stationary conditions:

|q_dot| < epsilon_v

A sustained parameter/current shift while moving should not automatically be called payload.

LATENCY METHODOLOGY:
Treat network and physical-response latency as separate measurements.

METHOD 1:
CRI application-layer ACK RTT

RTT = (t_ack_rx - t_tx) * 1000

RTT/2 may be reported only as:
“estimated one-way communication delay”

because it assumes approximately symmetric forward/reverse delay.

METHOD 2:
Telemetry-observed motion response.

GUI_to_TX =
(t_tx - t_gui) * 1000

TX_to_Motion =
(t_motion - t_tx) * 1000

GUI_to_Motion =
(t_motion - t_gui) * 1000

t_motion is based on motion observed in incoming CRI STATUS telemetry.

Do NOT call this pure actuator latency.

NETWORK RTT AND MOTION-RESPONSE SAMPLES SHOULD BE TREATED AS INDEPENDENT DATASETS UNLESS AN EXPERIMENT EXPLICITLY CORRELATES THEM.

MANUSCRIPT STRUCTURE:

- Introduction

  - fixed-threshold limitations in polymer-joint cobots

  - thermal/friction drift

  - sensorless context-aware DT contribution

- System Architecture & Latency Validation

  - ReBeL hardware

  - CRI

  - Python DT

  - Method 1 RTT

  - Method 2 telemetry-observed motion response

- Dynamic Kinematic Modeling & Adaptive Baseline

  - scientifically justified multi-DOF regressor

  - acceleration if experimentally justified

  - gravity/cross-axis coupling

  - true VFF-RLS

  - residual model

- Context-Aware Disambiguation

  - thermal drift

  - transient collision

  - soft contact

  - payload/grasp state

  - stationary/context gating

- Experimental Validation

  - ~45 min thermal burn-in

  - static baseline vs fixed-FF RLS vs VFF-RLS

  - collision experiments under cold/warm conditions

  - ROC / precision / recall / F1 / confusion matrices

  - context-aware multi-class confusion matrix

  - payloads 0 g / 100 g / 250 g / 500 g

  - payload RMSE / MAE / bias / standard deviation

- Conclusion

  - adaptive supervisory Digital Twin

  - standard API telemetry

  - no external force/torque sensor

  - complements existing safety system

NEXT IMPLEMENTATION PRIORITY:
Do NOT continue spending effort optimizing the GUI or latency pipeline unless a concrete experimental problem appears.

Next work should be:

- mathematically define the VFF law lambda(t);

- define the final regressor phi;

- define state-machine states and transition equations;

- define stationary and grasp criteria;

- define soft-contact vs collision vs payload rules;

- define experimental hypotheses and calibration procedures;

- only then ask Copilot to implement those exact equations.

IMPORTANT:
Do not let Copilot invent the VFF formula or scientific classification rules. They are part of the research contribution and must first be defined deliberately and justified mathematically/experimentally.

When you come back in a new conversation, you can simply say: “Continue my Context-Aware Digital Twin research from the saved project context. Start with defining the VFF-RLS mathematics and state machine.”